# Rom Update Utility (RUU)

#### Getting Started

Before you run the ROM Update Utility (RUU), make sure you have done the following:

- The correct PC driver for your Android phone has been installed.

- The Android phone is connected to your PC using the USB cable. The PC must recognize the Android phone.

- Standby or hibernation mode of your PC has been disabled.

- Close all running applications on the PC.

Important Do not interact with your Android phone unless the RUU wizard instructs you to do so.

## General Procedure on Installing the ROM Update Utility (RUU)

- Installing the ROM Update Utility (RUU) will delete ALL information and data on your Android phone. We recommend that you back up your personal data from the Android phone to your PC by using PC Sync Tools.

- On your PC, run the ROM Update Utility (RUU) to install the ROM update to your Android phone. (The next section will provide you a detailed procedure on how to install the ROM Update).

- Restore your data back to your Android Phone using PC Sync Tools.

## How to Install the ROM Update

Warning: Installing the ROM Update Utility (RUU) will delete ALL information and data on your Android phone.

1.	On the Welcome screen, click Next to begin the RUU installation.

2.	Check the listed instructions in the dialog box and make sure you have done them. Click Next to proceed.

3.	A message will then be displayed indicating that the utility is verifying and acquiring information about the Android phone as preparation for the update process. Wait for this verification process to be finished.

4.	After the verification process, current information about the Android phone will be displayed. Click Update to proceed.

5.	Below the Android phone information, information about the current image version and the new image update version will be displayed. If you are sure that you want to install the new RUU, click Next to proceed.

6.	A confirmation message will appear and show the time it will take to install the necessary files (including radio image) to the Android phone. Click Next to begin the update. A progress bar will run through the screen during the update process.

Note: In the middle of the update process, the progress bar may stop moving. This is normal as the RUU is automatically adjusting itself to complete the update process.

7. When you see the screen below, it means the RUU update is complete. Click Finish to exit the utility.

## Troubleshooting and Error Recovery

Almost all RUU-related errors can be recovered. When an error occurs, the utility will display an error message and provide recovery measures to allow you to continue the update process.

If errors cannot be recovered, please refer to the Q&A section for more details.

ERROR [110]: FILE OPEN ERROR

This error message indicates that the RUU lacks some specific files or they are damaged, so the RUU cannot continue with the update. You have to get the complete RUU package and try again.

ERROR [120]: MAIN BATTERY POWER

This error message will appear when the Android phone’s battery power is not sufficient (Batter power should be more than 30%). Although RUU will instruct you to plug in the AC adapter, it will still need to make sure that your Android phone has enough power to complete the update. (The Android phone cannot charge the battery when it is connected to the PC while installing the RUU).

ERROR [130]: MODEL ID ERROR

ERROR [131]: CUSTOMER ID ERROR

One of these error messages will appear when you use the wrong RUU to do the update. The RUU will check if the Model ID and Language ID are compatible with the Android phone. Make sure you use the correct RUU to update.

ERROR [155~159]: IMAGE ERROR

One of these error messages will appear when you use the incorrect RUU to update and the image file size is larger than the Flash ROM size. In this case, download the correct RUU version and try again.

ERROR [170]: USB CONNECTION ERROR

This error message will appear when the Android phone is not correctly connected to the host PC. It might be a USB connection problem or a PC USB driver problem. Make sure the Android phone is connected to the host PC correctly and/or check if the PC driver matches the Android phone.

Q & A for RUU Update

1.	Q: What should I do before calling the service center for help?

A: Take note of the error message (error no.) shown on the setup screen of your PC so you can tell the service center person the error.

2.	Q: How long will it take for the verification process to finish?

A: It takes around 1 minute to finish the verification process but it also depends on the speed of your PC.

3.	Q: What should I do if I see the progress bar is not moving on the Android phone?

A: DON’T DO ANYTHING. This is normal as the RUU is adjusting itself to complete the process. You should not interrupt it by disconnecting the Android phone from the PC unless you are instructed to do so.

4.	Q: During the update, the progress bar is moving. If the error message (ERROR [150], [151]…[159]) appears, what should I do?
A: This may occur when the connection is lost, power is lost, the computer host is down, or due to other unexpected reasons. You will see some information on the Android phone’s screen. If the progress bar is moving, it means the update is processing, and your Android phone will reboot automatically and the update is successful. If your Android phone's screen shows exclamation marks in four corners, it means the update has failed. The Android phone will not be able to go into the Android screen. This problem usually can be recovered. You need to follow the instructions in the error message box, which is to reset the Android Phone and run the RUU again.

5.	Q: During the update, the progress bar has stopped moving for a few minutes, but there is no error message, what should I do?

A: You can disconnect your Android phone from the USB cable first to see if there’s any error message that appears. If there’s an error message, please follow the instructions to try to recover. If there’s no error message, please reset the Android phone and run the RUU again. If the problem persists, reset your PC and try again.

6.	Q: If I encounter ERROR [170]: USB CONNECTION ERROR, what should I do?

A: You can reset the Android phone and run the RUU again. If the problem persists, reset your PC and try again.

7.	Q: If I encounter ERROR [120]: LOW MAIN BATTERY POWER, what should I do?

A: This error message appears when your Android phone’s battery power is not sufficient (Battery power should be more than 30%.) Although the RUU will instruct you to plug the phone to the AC adapter, it still has to make sure that your Android phone has enough power to update the ROM. The Android phone battery cannot be charged while doing the RUU update.

8.	Q: I used RUU to update and it failed. Now, my Android phone always boots in the RUU failed mode and cannot enter the Android Home screen anymore, what should I do?
A: This is normal and can be recovered. Follow the instructions below to reset your Android phone and to establish a connection with the PC.

1.	Disconnect your Android phone from the USB cable.

2.	Reset your Android phone (Remove and reinstall the battery).

3.	Reconnect your Android Phone to the USB cable.

4.	Check if the Android phone is recognized by the PC.

5.	Return to the ROM Update Utility.